Setting up scanner as Access point

setup get from this website:

https://learn.pi-supply.com/make/how-to-setup-a-wireless-access-point-on-the-raspberry-pi/

- copy /etc/network/interfaces  to : /etc/network/interfaces.d

- hash/delete/ everything except

# Include files from /etc/network/interfaces.d:

source-directory /etc/network/interfaces.d

in original file in folder interfaces

- in file /etc/network/interfaces .d/interfaces hash wifi lines as following:

#FOR WIFI UNCOMMENT BELOW a can leave above as is.

#note that UI shows wifi as off but actually it is working.

#auto wlan0

allow-hotplug wlan0

iface wlan0 inet dhcp

#        wpa-conf /etc/wpa_supplicant/wpa_supplicant.conf

wireless-power on

#wireless-power off

#iface default inet dhcp

#END WIFI

- install AP and  dns –

- sudo apt install dnsmasq hostapd

- Since the configuration files are not ready yet, turn the new software off as follows:

sudo systemctl stop dnsmasq

sudo systemctl stop hostapd

- To configure the static IP address, edit the dhcpcd configuration file with:

sudo nano /etc/dhcpcd.conf

Insert following:

interface wlan0

static ip_address=192.168.4.1/24

nohook wpa_supplicant

- Now restart the dhcpcd daemon and set up the new wlan0 configuration:

- sudo service dhcpcd restart

- Configuring the DHCP server (dnsmasq)

The DHCP service is provided by dnsmasq. By default, the configuration file contains a lot of information that is not needed, and it is easier to start from scratch. Rename this configuration file, and edit a new one:

sudo mv /etc/dnsmasq.conf /etc/dnsmasq.conf.orig

sudo nano /etc/dnsmasq.conf

Type or copy the following information into the dnsmasq configuration file and save it:

interface=wlan0      # Use the require wireless interface - usually wlan0

dhcp-range=192.168.4.2,192.168.4.20,255.255.255.0,24h

### 11 	Configuring the access point host software (hostapd)

You need to edit the hostapd configuration file, located at /etc/hostapd/hostapd.conf, to add the various parameters for your wireless network. After initial install, this will be a new/empty file.

sudo nano /etc/hostapd/hostapd.conf

interface=wlan0

driver=nl80211

ssid=NameOfNetwork

hw_mode=g

channel=7

wmm_enabled=0

macaddr_acl=0

auth_algs=1

ignore_broadcast_ssid=0

wpa=2

wpa_passphrase=AardvarkBadgerHedgehog

wpa_key_mgmt=WPA-PSK

wpa_pairwise=TKIP

rsn_pairwise=CCMP

12	We now need to tell the system where to find this configuration file.

sudo nano /etc/default/hostapd

Find the line with #DAEMON_CONF, and replace it with this:

DAEMON_CONF="/etc/hostapd/hostapd.conf"

13	start manually services :

Sudo systemctl daemon-reload

Sudo systemctl start dnsmasq

Sudo systemctl start dhcpcd

14	if scanner booting up too long and there is error – failed network services you need to disable dnsmasq and dhcpcd:

Sudo systemctl disable dnsmasq

Sudo systemctl disable dhcpcd