Setting up Wifi on Scanner:

Client Mode:

- Raspberry Pi2

Change file /etc/network/interfaces to :

auto eth0

iface eth0 inet static

address 169.254.71.46

netmask 255.255.0.0

gateway 169.254.30.155

2.	Raspberry Pi1

Change file /etc/network/interfaces to :

# interfaces(5) file used by ifup(8) and ifdown(8)

# Please note that this file is written to be used with dhcpcd

# For static IP, consult /etc/dhcpcd.conf and 'man dhcpcd.conf'

# Include files from /etc/network/interfaces.d:

source-directory /etc/network/interfaces.d

#ORIG is blank from here on.

#ip addresses from PC software to connect.

#in production, use same static ip as for version 1.

#NEW PI1 version2

#auto eth0

#iface eth0 inet static

#        address 169.254.30.155

#        netmask 255.255.0.0

#NEW PI2 version2

#auto eth0

#iface eth0 inet static

#        address 169.254.71.46

#        netmask 255.255.0.0

#TEST BRIDGING PC TO PI2

auto br0

#iface br0 inet dhcp

iface br0 inet static

address 169.254.30.154

netmask 255.255.0.0

bridge_ports eth0 usb0

bridge_stp off

bridge_fd 0

bridge_waitport 0

auto eth0

iface eth0 inet static

address 169.254.30.156

netmask 255.255.0.0

auto usb0

iface usb0 inet static

address 169.254.30.155

netmask 255.255.0.0

#FOR WIFI UNCOMMENT BELOW a can leave above as is.

#note that UI shows wifi as off but actually it is working.

auto wlan0

allow-hotplug wlan0

iface wlan0 inet dhcp

wpa-conf /etc/wpa_supplicant/wpa_supplicant.conf

wireless-power on

#wireless-power off

#iface default inet dhcp

#END WIFI

auto lo

iface lo inet loopback

up iptables-restore < /etc/iptables.ipv4.nat

3.	Setup wifi name and password :

Sudo nano /etc/wpa_supplicant/wpa_supplicant.conf

4.	Allow port forwarding:

sudo nano /etc/sysctl.conf

remove hash from net.ipv4.ip_forward1

5.	open ports 5000 and 5001:

Edit following file:

sudo nano /etc/iptables.ipv4.nat

paste this:

# Generated by xtables-save v1.8.2 on Mon Mar 15 15:51:30 2021

*filter

:INPUT ACCEPT [0:0]

:FORWARD ACCEPT [0:0]

:OUTPUT ACCEPT [0:0]

-A FORWARD -i wlan0 -o eth0 -p tcp -m tcp --dport 5000 --tcp-flags FIN,SYN,RST,ACK SYN -m conntrack --ctstate NEW -j ACCEPT

COMMIT

# Completed on Mon Mar 15 15:51:30 2021

# Generated by xtables-save v1.8.2 on Mon Mar 15 15:51:30 2021

*nat

:PREROUTING ACCEPT [0:0]

:INPUT ACCEPT [0:0]

:POSTROUTING ACCEPT [0:0]

:OUTPUT ACCEPT [0:0]

-A PREROUTING -i wlan0 -p tcp -m tcp --dport 5000 -j DNAT --to-destination 169.254.71.46:5005

-A PREROUTING -i wlan0 -p tcp -m tcp --dport 5001 -j DNAT --to-destination 169.254.30.155:5005

COMMIT

# Completed on Mon Mar 15 15:51:30 2021

6. restart and try if client working /for testing purposes its possible to change route table in windows :

169.254.30.155 mask 255.255.255.255 192.168.x.x  ----wifi ip address of scanner

169.254.71.46  mask 255.255.255.255 192.168.x.x ----wifi ip address of scanner

7. to make pi detected from windows

Sudo apt-get –y install samba

Samba wont start after boot so type:

Sudo systemctl start nmbd

I added this to /etc/rc.local

So it auto starts on boot.